# Relating Simulation Parameters to Experimental Photophysics & Benchmarking Guide

## Part 1: Relating BlinkOnRate/BlinkOffRate to Experimental Photoblinking

### The Physics of Photoblinking

Fluorescent molecules like Rhodamine B undergo photoblinking - stochastic transitions between emissive (ON) and non-emissive (OFF) states. The OFF state is typically:

• A triplet state (T₁) with lifetime ~1-10 ms

• A radical dark state formed through the triplet, with lifetime ~10 ms to seconds

• A charge-separated state from electron transfer

### Experimental Timescales for Rhodamine Dyes

| Parameter | Rhodamine 6G/B | Units | Source |
| --- | --- | --- | --- |
| Fluorescence lifetime | 1.5-4 ns | ns | Varies with solvent |
| Triplet lifetime | 1-10 ms | ms | In polymer matrices |
| Dark state lifetime | 10 ms - 10 s | ms-s | Oxygen-dependent |
| ON time (τ_on) | 100 ms - 10 s | ms-s | Environment-dependent |
| OFF time (τ_off) | 1 ms - 1 s | ms-s | Environment-dependent |

### Converting Experimental Values to Simulation Parameters

The simulation uses per-frame probabilities:

• BlinkOffRate = Probability of turning OFF per frame (when currently ON)

• BlinkOnRate = Probability of turning ON per frame (when currently OFF)

Key relationships:

BlinkOffRate ≈ FrameTime / τ_on    (probability of turning off per frame)

BlinkOnRate  ≈ FrameTime / τ_off   (probability of recovering per frame)

### Example Calculations

For Rhodamine B in PVA (typical SMOLM conditions):

• τ_on ≈ 500 ms (mean time in ON state)

• τ_off ≈ 70 ms (mean dark state duration)

• FrameTime = 50 ms

BlinkOffRate = 50 ms / 500 ms = 0.10   % 10% chance to turn OFF per frame

BlinkOnRate  = 50 ms / 70 ms  = 0.71   % 71% chance to turn ON per frame

These are the defaults in run_demo.m!

### Parameter Regimes

| Regime | BlinkOffRate | BlinkOnRate | Physical Meaning |
| --- | --- | --- | --- |
| Stable emitter | 0.01-0.05 | 0.8-0.95 | Long ON times, quick recovery |
| Moderate blinking | 0.10-0.20 | 0.5-0.7 | Typical organic dyes |
| Heavy blinking | 0.3-0.5 | 0.3-0.5 | STORM/PALM probes |
| Mostly dark | 0.5-0.8 | 0.1-0.2 | Photoactivatable probes |

### Factors Affecting Experimental Blinking

1. Oxygen concentration: Oxygen quenches triplet states → shorter τ_off → higher BlinkOnRate

2. Excitation intensity: Higher intensity → more triplet formation → more blinking

3. Polymer matrix: Restricts oxygen diffusion → longer dark states → lower BlinkOnRate

4. Temperature: Higher T → faster recovery → higher BlinkOnRate

5. Reducing agents: (e.g., BME, MEA in STORM) → control blinking rates

## Part 2: Benchmarking Parameters from ISBI Challenge & u-track3D

### ISBI Particle Tracking Challenge Parameters

The ISBI 2012 Challenge established standard benchmarking conditions:

Signal-to-Noise Ratio (SNR):

| Level | SNR Value | Description |
| --- | --- | --- |
| Very Low | 1 | Methods fail significantly |
| Low | 2 | Challenging detection |
| Critical | 4 | Threshold where methods break down |
| High | 7 | Good performance expected |

Particle Density:

| Level | Particles | Description |
| --- | --- | --- |
| Low | 50-100 | Well-separated, easy linking |
| Medium | 100-500 | Moderate ambiguity |
| High | 1000+ | Dense, significant overlap |

Motion Scenarios:

1. Vesicles: Brownian motion, D = 0.1-1 µm²/s

2. Receptors: Confined diffusion

3. Viruses: Directed + Brownian (constant velocity + diffusion)

4. Microtubule tips: Linear motion with variable velocity

### Performance Metrics

| Metric | Description | Formula |
| --- | --- | --- |
| α (alpha) | Detection precision | TP / (TP + FP) |
| β (beta) | Detection recall | TP / (TP + FN) |
| JSC | Jaccard similarity (tracks) | Overlap measure |
| JSCθ | Jaccard with temporal weighting | Time-weighted overlap |
| RMSE | Localization error | √(mean squared position error) |

## Part 3: Parameters to Modify for Benchmarking in run_demo.m

### Currently Available in generate6DTrajectories.m

Photophysics (Blinking/Bleaching):

'Signal', 1500, ...             % [photons] Higher = better SNR

'SignalStd', 300, ...           % [photons] Signal variation

'BlinkOffRate', 0.10, ...       % [prob/frame] 0.01-0.5

'BlinkOnRate', 0.70, ...        % [prob/frame] 0.1-0.95

'BleachRate', 0.0005, ...       % [prob/frame] Permanent photobleaching

Localization Noise:

'AddNoise', true, ...           % Enable localization noise

'NoiseXY', 20, ...              % [nm] Lateral localization precision

'NoiseZ', 50, ...               % [nm] Axial localization precision

'NoiseTheta', 5, ...            % [degrees] Polar angle precision

'NoisePhi', 5, ...              % [degrees] Azimuthal angle precision

Motion Parameters:

'Dtrans', 500, ...              % [nm²/s] Translational diffusion

'Drot', 0.05, ...               % [rad²/s] Rotational diffusion

### Recommended Benchmark Test Matrix

Test 1: SNR Sweep (Localization Noise)

% Low SNR (challenging)

'Signal', 500, 'NoiseXY', 50, 'NoiseZ', 100, 'NoiseTheta', 15, 'NoisePhi', 15

% Medium SNR

'Signal', 1500, 'NoiseXY', 20, 'NoiseZ', 50, 'NoiseTheta', 5, 'NoisePhi', 5

% High SNR (easy)

'Signal', 5000, 'NoiseXY', 10, 'NoiseZ', 20, 'NoiseTheta', 2, 'NoisePhi', 2

Test 2: Particle Density Sweep

% Low density

'NumParticles', 3, 'FieldOfView', [0 6000 0 6000]

% Medium density

'NumParticles', 10, 'FieldOfView', [0 6000 0 6000]

% High density

'NumParticles', 30, 'FieldOfView', [0 6000 0 6000]

Test 3: Blinking Severity Sweep

% Minimal blinking (stable emitter)

'BlinkOffRate', 0.02, 'BlinkOnRate', 0.90

% Moderate blinking (default)

'BlinkOffRate', 0.10, 'BlinkOnRate', 0.70

% Heavy blinking (STORM-like)

'BlinkOffRate', 0.30, 'BlinkOnRate', 0.40

## Part 4: Suggested Benchmark Scenarios for 6D-SMOLM

### Scenario Matrix for Publication

| Test ID | SNR | Density | Blinking | D_trans | D_rot | Purpose |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | High | Low | Light | 500 | 0.05 | Baseline/validation |
| A2 | Med | Low | Light | 500 | 0.05 | SNR effect |
| A3 | Low | Low | Light | 500 | 0.05 | SNR limit |
| B1 | High | Med | Light | 500 | 0.05 | Density effect |
| B2 | High | High | Light | 500 | 0.05 | Density limit |
| C1 | High | Low | Heavy | 500 | 0.05 | Blinking effect |
| C2 | High | Low | Light | 2000 | 0.20 | Fast motion |
| D1 | Med | Med | Moderate | 500 | 0.05 | Realistic conditions |

## References

1. Zondervan et al. "Photoblinking of Rhodamine 6G in Poly(vinyl alcohol)" J. Phys. Chem. A (2003)

2. Chenouard et al. "Objective comparison of particle tracking methods" Nature Methods (2014)

3. Roudot et al. "u-track3D: Measuring, navigating, and validating dense particle trajectories" Cell Reports Methods (2023)

4. ISBI Particle Tracking Challenge: http://bioimageanalysis.org/track/